| CHEMICAL ENGINEERING TRANSACTIONS VOL. xxx, 2025 | A publication of |
| --- | --- |
|  | The Italian Association of Chemical Engineering Online at www.cetjournal.it |
| ### Guest Editors: Fabrizio Bezzo, Flavio Manenti, Gabriele Pannocchia, Almerinda di Benedetto Copyright © 2025, AIDIC Servizi S.r.l. ISBN 979-12-81206-17-5; ISSN 2283-9216 |  |

Energy and Exergy Evaluation of an Integrated Dark Fermentation and Microbial Electrolysis System for Sustainable Hydrogen Production

Júlio C. de C. Mirandaa,*, Edwin Zondervanb

a Federal Universityof Mato Grosso – Department of Transportation, Chemical and Mining Engineering, Cuiabá, MT - Brazil

b University of Twente – Sustainable Process Technology – PSE, Enschede - Netherlands

*julio.miranda@ufmt.br

Abstract: This study evaluates hydrogen production from crude glycerol and vinasse using integrated dark fermentation and microbial electrolysis. Aspen Plus simulations showed high efficiency, with 95% COD reduction and hydrogen yields of 6.43 kg/h (fermentation) and 95.80 kg/h (electrolysis). Exergy efficiencies exceeded 81%, highlighting the system's potential for converting waste into renewable energy and supporting green hydrogen development.

Introduction

Global energy demand is rising rapidly, while hydrocarbon reserves are depleting and causing severe environmental issues like global warming and biodiversity loss. This urgency has driven the search for sustainable energy alternatives to meet demands while reducing environmental harm (Alcaraz-Gonzalez et al., 2020). Hydrogen (H₂) is increasingly recognized as a key solution due to its versatility and potential as a renewable energy source. Advancements in production methods, including thermochemical, electrolytic, photochemical, and biological processes, have highlighted its potential. However, hydrogen production still predominantly relies on fossil fuels, which account for 96% of commercially available hydrogen and emit up to 13.6 kg of CO₂ per kg H₂ (Asrul et al., 2021). Transitioning to green hydrogen through renewable-powered electrolysis and using waste-based substrates are crucial steps toward sustainability (Noori et al., 2024).

Dark fermentation is a promising method for sustainable hydrogen production, using microbial catalysts to convert biomass into hydrogen under anaerobic conditions (García & Cammarota, 2018). Operating without sunlight, it enables continuous operation and simplifies bioreactor design while producing value-added by-products like volatile fatty acids and alcohols. This environmentally friendly approach offers low energy demand, mild operational conditions, and no inhibitory by-products, making it a practical alternative to traditional, energy-intensive methods.

Microbial electrolysis, another bioelectrochemical system (BES) technology, enables hydrogen production from organic wastes and wastewater. Using electroactive bacteria, microbial electrolysis cells (MECs) operate at significantly lower energy demands (33.2–117 kWh/kg H₂) compared to abiotic methods (170–995 kWh/kg H₂), making them cost-effective and efficient (Noori et al., 2024). This process supports clean hydrogen production and addresses waste management challenges, though microbial system complexity introduces performance variability due to microbial dynamics, reactor configuration, and operational conditions (Muddasar et al., 2021).

Crude glycerol and vinasse, byproducts of biodiesel and ethanol production, are rich in organic content and suitable for biofuel production. Crude glycerol, generated at 1 kg per 10 kg of biodiesel, reached 5.87 billion pounds by 2020 (Kumar et al., 2019), while vinasse, produced at 8–15 liters per liter of ethanol, accounted for over 370 billion liters in Brazil's 2022–2023 sugarcane ethanol harvest (Menezes et al., 2023). Co-fermentation processes improve hydrogen yields by balancing nutrients and diluting inhibitory compounds (Yang & Wang, 2017). Using these byproducts supports sustainable biofuel production and mitigates waste issues like soil salinization and groundwater contamination (Ribeiro et al., 2021).

This study simulates and analyzes hydrogen production via dark fermentation and microbial electrolysis using crude glycerol and vinasse as substrates. It evaluates process efficiency and sustainability, aiming to maximize hydrogen yield while minimizing energy use and environmental impact. Detailed modeling and exergy analysis provide insights into transforming waste streams into renewable energy, advancing sustainable biorefinery systems.

Methodology

Exergy Analysis

Exergy analysis is a fundamental and straightforward method for evaluating the usable energy within a process, providing insights into its efficiency and sustainability. This approach is rooted in the Second Law of Thermodynamics, enabling the estimation of not only external energy losses but also internal inefficiencies caused by the degradation of materials and energy due to entropy production (Heijden and Ptasinsky, 2012). By quantifying these losses, exergy analysis offers a comprehensive understanding of where and how energy dissipation occurs within a system. Figure 1 illustrates an exergy balance applied to a process, showcasing the distribution of energy and entropy across different units. Using this concept, it was possible to calculate the exergy losses for each process unit, highlighting specific areas where performance improvements can be targeted.

Figure 1: Exergy Analysis Schematics

Where: H is enthalpy, S is entropy, Q is heat, W is work, T is temperature; and the subscripts denote R (raw materials), P (products), Eff (effluents), and Env (environment). Figure 1 illustrates the balance, represented by Equations 1–9 below, incorporating the term Irr, which accounts for process irreversibilities (exergy losses).

| inEi+inEjch+inEkQ+inElW=outEm+outEnch+outEpQ+outEqW+Irr | (1) |
| --- | --- |
| Ei=HR,i-HR,i0-T0(SR,i-SR,i0) | (2) |
| Ejch=Ψch0.njc | (3) |
| EkQ=Qin1-TEnvTin | (4) |
| ElW=Win | (5) |
| Em=HP,m-HP,m0-TEnvSP,m-SP,m0+HEff,m-HEff,m0-TEnv(SEff,m-SEff,m0) | (6) |
| Ench=Ψch0.nnc | (7) |
| EpQ=Qout1-TEnvTout | (8) |
| EqW=Wout | (9) |

Exergy efficiency is defined in two ways: one based on the product exergy, specifically hydrogen (Equations 9 and 10) and carbon dioxide (Equation 10), and the other incorporating heat-derived exergy (Equations 11 and 12). In the latter, the right side of Equation 1 is adjusted to represent the total input exergy of the process.

| ηP=EHydrogenE+ | (9) | ηP=EHydrogen+ECO2E+ | (11) |
| --- | --- | --- | --- |
| ηO=EHydrogen+outEnQE+ | (10) | ηO=EHydrogen+ECO2+outEnQE+ | (12) |

In Equations 9–12, the term E+ replaces the right side of Equation 1, representing the total exergy input to the process.

Process Simulation

The simulation software used was ASPEN Plus v14.1. To model electrolyte systems, the thermodynamic packages ELECNRTL and ENRTL-RK were applied. ELECNRTL, suited for aqueous systems with detailed ionic interactions, was primarily used, while ENRTL-RK handled mixed electrolyte and non-electrolyte systems, modeling microbial electrolysis cells.

As shown in Figure 2, vinasse (VINASSE) (Kumar et al., 2019) and crude glycerol (CR-GLY), with variability in crude glycerol composition simulated using Monte Carlo methods (14-87% glycerol, 0.014-0.078% nitrogen, 0.2-5.47% NaCl, 0.93-6.34% ash, and 8.16-43.42% water) (Carrilho et al. 2016), are mixed (M-101) and pumped (P-101) to the fermentation area. The fermentation reactions were modeled based on the studies by Lo et al. (2007, 2013). The reactor feed was fixed to 50 gglycerol/L, with the vinasse input adjusted to align with the glycerol concentration. Eight parallel fermentation reactors (Figure 3a, modeled as R-CSTR) operated with Clostridium pasteurianum CH4, producing biogas (H₂ and CO₂) along with organic products, including 1,3-propanediol, butanol, ethanol, acetic acid, and butyric acid, under conditions of 35°C and atmospheric pressure.

Fermentation gases are combined and purified, while liquid products and some microorganisms move to the separation and recycling system (MO-REC) and then to microbial electrolysis. In the flash vessel (F-101), water is separated, and biogas enters a membrane system (S-101), recovering 99% hydrogen (Hawkes et al. 2007) and generating purified H₂ and CO₂ streams.

In microbial electrolysis, four reactors (Figure 3b) arranged in series process liquid fermentation products. Two conversion reactors (R-YIELD) simulate anode reactions (organic decomposition to CO₂ and H⁺) and cathode reactions (H₂ formation). A separation block (SEP) represents the Proton Exchange Membrane (PEM), enabling H⁺ ion passage. The resulting gases (CO₂ and H₂) are combined with purified fermentation streams, compressed (145 atm for H₂, 57 atm for CO₂), and cooled to ambient temperature, completing production and storage.

Results

Process

Table 1 presents the yields from dark fermentation and microbial electrolysis processes. In the fermentation stage, glycerol conversion achieved a remarkable average of 99.41% with a minimal standard deviation (0.06%), while glucose conversion reached 72.64% with a slightly higher variability (1.29%). The process generated 6.43 kg/h of H₂ and 160.83 kg/h of CO₂. In contrast, microbial electrolysis demonstrated a significant reduction in COD from 925.49 kg/h (initial) to 46.66 kg/h (final), reflecting substantial organic matter removal. Additionally, the microbial electrolysis process produced 95.80 kg/h of H₂, far surpassing the fermentation yield, along with 966.04 kg/h of CO₂.

Table 1: Dark Fermentation and Microbial Electrolysis Yield

| Fermentation | avg. | s.d | Microbial electrolysis | avg. | s.d. |
| --- | --- | --- | --- | --- | --- |
| Glycerol Conversion | 99.41% | 0.06% | Initial COD (kg/h) | 925.49 | 154.29 |
| Glucose Conversion | 72.64% | 1.29% | Final COD (kg/h) | 46.66 | 7.78 |
| H2 generated (kg/h) | 6.43 | 0.93 | H2 generated (kg/h) | 95.80 | 15.63 |
| CO2 generated(kg/h) | 160.83 | 23.34 | CO2 generated(kg/h) | 966.04 | 163.15 |

Figure 2: Main Process Flowchart

| (a) | (b) |
| --- | --- |

Figure 3: (a) Fermentation Reactor; (b) Microbial Electrolysis Cell Simulation Scheme.

Energy Analysis

Table 2 summarizes the energy balance across the main system, dark fermentation, and microbial electrolysis stages, presenting average energy flows and their variability. In the main system, significant energy demand is observed in CP-101 and CP-102 (4.39E+05 and 1.22E+05 kcal/h), while pumps (P-101 to P-103) and heat exchangers exhibit low consumption with minimal variability. The dark fermentation reactors (R-DF1 to R-DF8) exhibit consistent cooling requirements (~-3.18E+03 kcal/h) with negligible deviations, The dark fermentation reactors (R-DF1 to R-DF8) exhibit consistent cooling requirements (~-3.18E+03 kcal/h) with negligible deviations, reflecting robust operation and effectively dampening variability in the process. In microbial electrolysis, energy usage varies significantly across the reactors and membrane due to their configuration. It is essential to analyze the system as a single integrated cell when assessing energy performance. This analysis does not account for thermal effects or overpotential, which are common in practice. Instead, it focuses solely on the chemical energy required to decompose the molecules.

Table 2: Energy consumption per equipment

|  |  | Equipment | Energy(kcal/h) |  | Equipment | Energy(kcal/h) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | avg. | s.d. |  | avg. | s.d. |
| Main | W | P-101 | 9.78E+01 | 8.33E+00 | P-102 | 1.46E-01 | 2.21E-02 |
|  |  | P-103 | 9.71E+01 | 6.36E+04 | CP-101 | 4.39E+05 | 6.36E+04 |
|  |  | CP-102 | 1.22E+05 | 1.03E+03 |  |  |  |
|  | Q | H-101 | 6.73E+04 | 1.21E+04 | SEP-101 | -4.16E+02 | 5.99E+01 |
|  |  | F-101 | -5.70E+03 | 8.24E+02 | CO-101 | -4.50E+05 | 2.22E+04 |
|  |  | CO-102 | -1.44E+05 | 2.22E+04 | MO-REC | 3.19E-01 | 3.85E-02 |
| Dark Fermentation | Q | R-DF1 | -3.18E+03 | 4.71E+02 | R-DF2 | -3.18E+03 | 4.71E+02 |
|  |  | R-DF3 | -3.18E+03 | 4.71E+02 | R-DF4 | -3.18E+03 | 4.71E+02 |
|  |  | R-DF5 | -3.18E+03 | 4.71E+02 | R-DF6 | -3.18E+03 | 4.71E+02 |
|  |  | R-DF7 | -3.18E+03 | 4.71E+02 | R-DF8 | -3.18E+03 | 4.71E+02 |
| Microbial Electrolysis | Q | ME-AN01 | 3.00E+05 | 4.82E+04 | ME-AN02 | 1.92E+05 | 3.18E+04 |
|  |  | ME-MB01 | 2.30E+04 | 3.94E+03 | ME-MB02 | 6.78E+03 | 1.17E+03 |
|  |  | ME-CA01 | 6.80E+03 | 1.51E+03 | ME-CA02 | 3.89E+02 | 1.63E+02 |
|  |  | ME-AN03 | 5.10E+01 | 2.93E+01 | ME-AN04 | 4.18E+04 | 6.89E+03 |
|  |  | ME-MB03 | 2.41E+03 | 4.30E+02 | ME-MB04 | 9.57E+02 | 1.65E+02 |
|  |  | MB-CA03 | 1.21E+02 | 3.61E+01 | ME-CA04 | 1.93E+02 | 4.08E+01 |

Exergy Analysis

Table 3 presents the exergy analysis and process efficiencies, highlighting the distribution and performance of input and output exergy. Despite considerable variability in crude glycerol composition at the inlet the final exergy does not exhibit wide fluctuations. Table 3 presents the exergy analysis and process efficiencies, highlighting the distribution and performance of input and output exergy. The total input exergy (E⁺) is 3.56E+09 kcal/h, with chemical exergy (E^ch) as the largest contributor (3.53E+09 kcal/h). The total output exergy (E⁻) is 3.42E+09 kcal/h, resulting in a net exergy loss (ΔE) of 1.38E+08 kcal/h. Efficiencies for hydrogen production and combined hydrogen-CO₂ production reach 81.92% and 82.25%, respectively, with minimal variability (~1.85–1.86%)..

Table 3: Exergy Analysis

| Exergy (kcal/h) |  |  |  |  |  | Efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | avg. | s.d. |  | avg. | s.d. |  | avg. | s.d. |
| E+ | 3.56E+09 | 5.29E+08 | E | 5.85E+04 | 1.07E+05 |  |  |  |
|  |  |  | EQ | 2.18E+03 | 3.91E+02 |  |  |  |
|  |  |  | EW | 5.61E+05 | 9.19E+04 | ηEHydrogen/E+ | 81.92% | 1.85% |
|  |  |  | Ech | 3.53E+09 | 5.92E+08 | η(EHydrogen+EQ)/E+ | 81.92% | 1.85% |
| E- | 3.42E+09 | 5.15E+08 | E | -6.81E+04 | 1.03E+05 | η(EHydrogen+ECO2)/E+ | 82.25% | 1.86% |
|  |  |  | EQ | -3.80E+03 | 6.28E+02 | η(EHydrogen+ECO2+EQ)/E+ | 82.25% | 1.86% |
|  |  |  | EW | 0.00E+00 | 0.00E+00 |  |  |  |
|  |  |  | Ech | 3.42E+09 | 5.77E+08 |  |  |  |
| ΔE | 1.38E+08 | 1.44E+07 |  |  |  |  |  |  |

Conclusions

The analysis of dark fermentation and microbial electrolysis processes underscores the system's exceptional robustness and efficiency in hydrogen production from waste substrates such as glycerol and vinasse. Energy and exergy assessments confirm high performance, showcasing the system's capability to maximize resource utilization. Monte Carlo simulations applied to raw materials reveal minimal variability in key parameters, affirming system stability, while a net exergy loss of 1.38E+08 kcal/h highlights clear opportunities for further optimization. This integrated approach highlights the potential to transform waste streams into clean energy and contributes to the advancement of more sustainable and efficient biorefinery systems.

Acknowledgments

The authors gratefully acknowledge financial support from the National Council for Scientific and Technological Development (CNPq), Brazil, under grant number 200132/2024-5.

References

Alcaraz-Gonzalez, Rodriguez-Valenzuela, G., & Gomez-Martinez, J. J., 2020, Hydrogen production automatic control in continuous microbial electrolysis cells reactors used in wastewater treatment, Journal of Environmental Management, p. 111869.

Asrul, M. A., Atan, M. F., Yun, H. A., & Lai, J. C., 2021, Mathematical model of biohydrogen production in microbial electrolysis cell: A review. International Journal of Hydrogen Energy, pp. 37174-37191.

Noori, M. T., Rossi, R., Logan, B. E., & Min, B. (July de 2024). Hydrogen production in microbial electrolysis cells with biocathodes. Trends in Biotechnology, pp. 815-828.

García, A. B., & Cammarota, M. C. (29 de November de 2018). Biohydrogen production from pretreated sludge and synthetic and real biodiesel wastewater by dark fermentation. International Journal of Energy Research, pp. 1586-1596.

Muddasar, M., Liaquat, R., Aslam, A., Rahamn, M. Z., Abdullah, A., Khoja, A. H., . . . Bahadar, A. (15 de December de 2021). Performance efficiency comparison of microbial electrolysis cells for sustainable production of biohydrogen - A comprehensive review. International Journal of Energy Research, pp. 5625-56245.

Kumar, L. R., Yellapu, S. K., Tyagi, R. D., & Zhang, X. (13 de September de 2019). A review on variation in crude glycerol composition, bio-valorization of crude and purified glycerol as carbon source for lipid production. Bioresource Technology, p. 122155.

Menezes, C. A., Almeida, P. S., Camargo, F. P., Dalforno, T., Oliveira, V. M., Sakamoto, I. K., . . . Silva, E. L. (14 de August de 2023). One versus two-stage codigestion of sugarcane vinasse and glycerol: Assessing combinations at mesophilic and (hyper) thermophilic conditions. Science of the Total Environment, p. 166294.

Yang, G., & Wang, J. (24 de August de 2017). Enhanced Hydrogen Production from Sewage Sludge by Co-fermentation with Forestry Wastes. Energy & Fuels, pp. 9633-9641.

Ribeiro, J. C., Mota, V. T., Oliveira, V. M., Decanal, G. C., & Zaiat, M. (24 de December de 2021). Hydrogen and organic acid production from dark fermentation of sugarcane vinasse withou buffers in mesophilic and thermophilic conditions. Journal of Chemical Technology & Biotechnology, pp. 1585-1596.

H. van der Heijden, K.J. Ptasinski (2012). Exergy analysis of thermochemical ethanol production via biomass gasification and catalytic synthesis. Energy, 46, 200-210.

Carrilho, E. N. V. M., Labuto, G., & Kamogawa, M. Y. (2016). Destination of vinasse, a residue from alcohol industry: Resource recovery and prevention of pollution. Environmental Materials and Waste: Resource Recovery and Pollution Prevention, 21–40

Kumar, L. R., Yellapu, S. K., Tyagi, R. D., & Zhang, X. (2019). A review on variation in crude glycerol composition, bio-valorization of crude and purified glycerol as carbon source for lipid production. Bioresource Technology, 293, 122155.

Lo, Y., Chen, W., Hung, C., Chen, S., & Chang, J. (7 de September de 2007). Dark H2 fermentation from sucrose and xylose uding H2-producing indigenous bacteria: Feasibility and kinetic studies. Water Research, pp. 827-842.

Lo, Y., Chen, X., Huang, C., Yuan, Y., & Chang, J. (17 de June de 2013). Darnk fermentative hydrogen production with crude glycerol from biodiesel industry using indigenous hydrogen-producing bacteria. International Journal of Hydrogen Energy, pp. 15815-15822.

Hawkes, F. R., Hussy, I., Kyazze, G., Dinsdale, R., & Hawkes, D. L. (2007). Continuos dark fermentative hydrogen production by mesophilic microflora: Priciples and progress. International Journal of Hydrogen Energy, 32, pp. 172-184.